|  | Harley-Davidson Recommended |  |
| --- | --- | --- |
| 2500 MILE SERVICE |  |  |

Road King

Inspect engine oil level.

Inspect air cleaner and service as required

Inspect transmission lubricant level

Check and adjust drive belt (if needed)

Inspect brake pads and discs for wear

Inspect oil lines and brake systems for leaks

Check, adjust operation of enrichener.*

Inspect fuel valve*, lines and fittings for leaks

Check operation of all electrical equipment and switches

Check battery electrolyte level and clean connections

Check tire pressure and inspect tread

Road test. Date of Service _________   Mileage ________ By ____________________

* If applicable