|  | Harley-Davidson Recommended |  |
| --- | --- | --- |
| 5000 MILE SERVICE |  |  |

Road King

Change engine oil & filter.  (3.5 – 4 Quarts )

Inspect air cleaner and service as required

Change primary chaincase lubricant and clean magnetic drain plug

Check/adjust primary chain

Check clutch adjustment

Change transmission lubricant and clean the magnetic drain plug   (20-24 oz.)

Check and adjust belt

Lube the foot shift/brake lever bearings*

Check rear brake pedal adjustment

Inspect brake pads and discs for wear

Check brake fluid reservoir levels and condition

Inspect oil lines and brake systems for leaks

Lubricate the brake & clutch lever, throttle cables, throttle, clutch cable and jiffy stand

Check, adjust operation of enrichener.*

Inspect fuel valve*, lines and fittings for leaks

Check operation of all electrical equipment and switches

Check battery electrolyte level and clean connections

Check tire pressure and inspect tread

Check wheel spoke tightness.*

Inspect, adjust, repack front fork bearing (Lube at 5,000 intervals - Disassemble at 50,000)

Repack rear fork bearings.*

Check condition of rear shock absorbers

Check air suspension -- pressure, operation and leakage.*

Check tightness of all fasteners except engine head bolts

Lubricate hinges, latches -- fuel door, Tour-Pak, saddlebags.*

Check ignition timing and vacuum hose.*

Check engine idle speed adjustment

Change spark plugs (10,000 Miles)

Check stabilizer links and engine mounts (10,000 Miles)

Clean fuel tank filter screen (carb. only). (20,000 Miles)

Change front fork oil.* (20,000 Mile Intervals) (11.1oz each side)

Road test. Date of Service _________   Mileage ________ By ____________________

* If applicable